Estimated cost saving analysis

- One manual inspector costs around ₹20,000.

- While the system is one time investment, it does requires electricity and maintenance cost. The monthly cost for electricity and maintenance is ₹2,000 and yearly cost is ₹24,000.

If automation reduces one manual inspector operator, then

Monthly Savings=₹18,000

Annual Savings=₹2,16,000

Return on Investment

- The prototype implementation cost is ₹ 15,000.

- The monthly savings is ₹ 18,000 so investment recovery happens in about 25 days.

Manual human inspection accuracy is approximately 85% to 90%.

While the YOLO v8n model which was trained on a custom dataset of approximately 1000 images has accuracy of defect detection of 94.6%.

Yolo v8n model has 3.2 million parameters

Defect detection:

- Precision = 87.2%

- Recall = 96%

- mAP@50 = 94.6%

Gear detection:

- Precision = 97.8%

- Recall = 100%

- mAP@50 = 99.5%

Total Cost for the Prototype is ₹ 8770

| Raspberry Pi 4 | 4000 |
| --- | --- |
| L shape bracket | 200 |
| DC motor | 150 |
| mg995 servo | 300 |
| conveyor belt | 290 |
| camera | 1000 |
| power supply 12 volts | 300 |
| raspi power supply | 800 |
| 3d printed parts | 600 |
| pvc pipes | 100 |
| sand paper | 30 |

- On Average, manual inspection for defects takes 5 seconds while automated inspection using YOLO takes 1 second.

- This shows that inspection time is reduced by 5x which saves time in the manufacturing process thus increasing the productivity.

- The time taken by the rejection mechanism to push out defected objects is 2 seconds.

Throughput

- Manual checks require 5 seconds per gear

- Manual=12 gears/min

- Automation=60 gears/min

- Therefore the throughput increases nearly 4x to 5x